Command Reference for  pTPOSdll.DLL

# Initial / Terminate the TPOS Object

procedure TPOSOpen

Before you can call the commands of TPOS, you must call this command.

procedure TPOSClose

After you call this command, all the command will disabled.

# LCD control command

procedure SetLCDEnabled(vOn : boolean)

Enable / Disable LCD

procedure SetLCDBackLight(BackLight : byte)

set the back light

procedure LCDSaveContrast

Save the setting of contrast

# Customer Display control command

procedure SetCustomerDisplayEnabled(vOn : boolean)

Enable / Disable the Customer Display

procedure SetCustomerDisplayContrast(Contrast : byte)

Set the contrast

procedure SetCustomerDisplayBackLight(BackLight : byte)

Set the back light

procedure CustomerDisplaySaveContrast

Save the setting of contrast

procedure CustomerDisplaySaveBackLight

Save the setting of back light

procedure SetCustomerDisplayON(vOn : boolean)

Set the Customer Display ON / OFF

procedure SetCustomerDisplayCursorOn(vOn : boolean)

Set the cursor ON / OFF

procedure SetCustomerDisplayBlinkOn(vOn : boolean)

Set the blinker ON / OFF

procedure CustomerDisplayClear

Clean up the Customer Display and set the cursor to the position (1,1)

procedure CustomerDisplayCursorHome

Set the cursor to the position (1,1)

procedure CustomerDisplaySetCursor(X,Y : byte)

Set the cursor to the position (X,Y)

procedure CustomerDisplayShowString(Str : string)

Show a string to the Customer Display

# MCR control command

procedure SetMCREnabled(vOn : boolean)

Enable / Disable the MCR

procedure SetMCRTrack1Enabled(vOn : boolean)

Enable / Disable the Track 1 of MCR

procedure SetMCRTrack2Enabled(vOn : boolean)

Enable / Disable the Track 2 of MCR

procedure SetMCRTrack3Enabled(vOn : boolean)

Enable / Disable the Track 3 of MCR

function GetMSRData : string

Get the data from the MCR receive buffer and purge it